{Business Name}  RESPIRATORY PROTECTION PROGRAM

PURPOSE

{Business Name} has determined that certain employees are exposed to respiratory hazards during routine operations.  These hazards include chemical vapors, certain biohazards, and particulates.  The purpose of this program is to ensure that all {Business Name} employees are protected from exposure to these respiratory hazards.

POLICY

Engineering controls, such as ventilation and substitution of less toxic materials, are the first line of defense at {Business Name} however, engineering controls and administrative controls are not always feasible for some of our operations, or do not always completely control the identified hazards.  In these situations, respirators and other protective equipment must be used.  Respirators are also needed to protect employees’ health during emergencies.  The work activities requiring respirator use at {Business Name} are outlined in Table 1 in the Scope and Application section of this program.

In addition, some employees have expressed a desire to wear respirators during certain operations that do not require respiratory protection.  As a general policy, {Business Name} will review each of these requests on a case-by case basis.  As outlined in the Voluntary Respirator Use Program and in the Scope and Application section of this program, voluntary respirator use is subject to certain requirements.

SCOPE AND APPLICATION

This program applies to all employees who are required to wear respirators during normal work operations, and during some non-routine or emergency operations such as a spill of a hazardous substance.  All employees working in these areas and engaged in certain processes or tasks must be enrolled in the {Business Name} respiratory protection program.

Table 1:  Work activities that require respirator protection.

	WORK PROCESS
	LOCATION
	TYPE OF RESPIRATOR

	
	
	

	
	
	

	
	
	

	
	
	


RESPONSIBILITIES

Program Administrator

The Program Administrator is responsible for administering the respiratory protection program.  Duties of the program administrator include:

· Identifying work areas, processes or tasks that require workers to wear respirators, and evaluating hazards.

· Selection of respiratory protection options

· Monitoring respirator use to ensure that respirators are used in accordance with their certifications

· Arranging for and/or conducting training.

· Ensuring proper storage and maintenance of respiratory protection equipment.

· Conducting/supervising qualitative fit testing.

· Ensuring medical surveillance program completion.

· Maintaining training/fit test records required by the program.

· Evaluating and updating the program as needed.

The Program Administrator for {Business Name} has been designate in {Program Administrator Name}.

Supervisors/Team Leaders/Department Directors

Supervisors/Team Leaders/Department Directors are responsible for ensuring that the respiratory protection program is implemented in their particular areas.  In addition to being knowledgeable about the program requirements for their own protections, supervision must also ensure that the program is understood and followed by the employees under their charge.  Duties of supervision include:

· Ensuring that employees under their supervision (including new hires) have received appropriate training, fit testing and annual medical evaluation.

· Ensuring the availability of appropriate respirators and accessories.

· Awareness of tasks that require the use of respiratory protection.

· Enforcing the proper use of respiratory protection when necessary.

· Ensuring that respirators are properly cleaned, maintained, and stored according to the respiratory protection plan.

· Ensuring that respirators fit well and do not cause discomfort.

· Continuous monitoring of work areas and operations to identify respiratory hazards.

· Coordination with the Program Administrator on how to address respiratory hazards or other concerns regarding the program.

Employees

Each employee has the responsibility to wear his or her respirator when and where required and in the manner in which they were trained.  Employees must also:

· Wear the respirator for approved locations only.

· Care for and maintain their respirators as instructed, and store them in a clean sanitary location.

· Inform their supervisor or {fit tester} if the respirator no longer fits well, and request a new one that fits properly.

· Inform their supervisor or {fit tester} of any respiratory hazards that they feel are not adequately addressed in the workplace and other concerns they may have regarding the program.

PROGRAM ELEMENTS

Selection Procedures

The Program Administrator will select respirators to be used on site based on the hazards to which workers are exposed, and in accordance with all OSHA standards and NIOSH guidelines.

{Title for person responsible} will conduct a hazard evaluation for each operation, process, or work area where airborne contaminants may be present in routine operations or during an emergency.  The hazard evaluation will include:

· Identification and development of a list of hazardous substances used in the workplace by department or work process.

· Review of work processes to determine where potential exposures to the hazardous substances identified may occur.  This review shall be conducted by surveying the workplace, reviewing process records, and talking with employees and supervisors.

The hazard evaluation may include exposure monitoring to quantify potential hazardous exposures.  Monitoring will be conducted if the person conducting the evaluation determines that it is required.

NIOSH Certification

All respirators must be certified by the National Institute for Occupational Safety and Health (NIOSH) and shall be used in accordance with the terms of that certification.  Also, all filters, cartridges, and canisters must be labeled with the appropriate NIOSH approval label.  The label must not be removed or defaced while it is in use.

Emergency Use

Supplied air and air purifying respirators will be used depending on the emergency.  Appropriate respiratory protection will be selected by the Incident Commander or the Safety Officer.

Voluntary Respirator Use

Disposable N95 respirators are the only Voluntary Use respirator permitted by {Business Name}.  {Business Name} does not permit its employees to use their own respirators.  All parts of the {Business Name}  Voluntary Respirator Use Program, which is contained in Attachment A of this document, must be followed.

{Business Name}  may provide disposable N95 respirators at no charge to employees for voluntary use for working around dust generating operations or nuisance levels of chemical particulates.

All employees who voluntarily choose to wear respirators will be provided with a copy of Attachment D of this program.  Attachment D details the requirements for voluntary use of respirators by employees.

{Person responsible} shall authorize voluntary use of respiratory protective equipment on a case-by-case basis, depending on specific workplace conditions and the results of the medical evaluations.  Voluntary use will be determined by OSHA Respirator use requirements.

Medical Evaluation

Employees who are required to wear respirators, or have special permission by {individual} to voluntarily wear a reusable APR, must pass a medical exam before being permitted to wear a respirator on the job.  Employees are not permitted to wear respirators until {individual} has determined that they are medically able to do so.  Any employee refusing the medical evaluation will not be allowed to work in an area requiring respirator use.

{Entity or person} will provide the medical evaluations.  Medical evaluation procedures are as follows:

· Employee’s supervisor will contact {Person responsible}.

· {Person responsible} will send out a medical evaluation questionnaire.
· Employee must be allowed to fill out the medical evaluation on work time and in a confidential manner.

· To the extent feasible, {Business Name} will assist employees who are unable to read the questionnaire.

· {Individual responsible} will make a determination as to the extent of the physical examination.

· Follow-up medical exams will be granted to employees as required by the standard, and/or as deemed necessary by {individual responsible}.

· All employees will be granted the opportunity to speak with {medical personnel} about their medical evaluation, if they so request.

After an employee has received clearance and starts to wear his or her respirator, additional medical evaluations will be provided under the following circumstances:

· Employee reports signs and/or symptoms related to their ability to use a respirator, such as shortness of breath, dizziness, chest pains, or wheezing.

· {Individual in facility} informs the Program Administrator that the employee needs to be reevaluated.

· Information from the program, including observations made during fit testing and program evaluation, indicates a need for reevaluation.

· A change occurs in workplace conditions that may result in an increased physiological burden on the employee.

All examinations and questionnaires are to remain confidential between the employee and {provider}.

Fit Testing

Fit testing is required for {Business Name} employees who are required to wear respirators.

Employees who are required to wear a respirator will be fit tested:

· Prior to being allowed to wear any respirator.

· Annually thereafter.

· When there are changes in the employee’s physical condition that could affect respiratory fit (e.g., obvious change in body weight, facial scarring, etc.).

Employees will be fit tested with the make, model and size of respirator that they will actually wear.  Employees will be provided with several models and sizes of respirators so that they may find an optimal fit.

Fit testing will be conducted following the OSHA approved Qualitative Fit Test Protocol (QLFT)

General Use Procedures

Employees will use their respirators under conditions specified by this program, and in accordance with the training they receive on the use of each particular model.  In addition, the respirator shall not be used in a manner for which it is not certified by NIOSH or by its manufacturer or for an exposure or work activity that is not approved by {Individual responsible}.

All employees shall conduct user seal checks each time they wear their respirator.  Employees shall use either the positive or negative pressure check (depending on which test works best for them).

All employees shall leave a potentially hazardous work area to clean their respirator if the respirator is impeding their ability to work, change filters or cartridges, replace parts, or to inspect respirator if it stops functioning as intended.  Employees should notify their supervisor of any respirator problems.

Respirator Malfunction

For any malfunction a respirator (e.g., such as breakthrough, facepiece leakage, or improperly working valve), the wearer of respirator should inform his or her supervisor that the respirator no longer functions as intended, obtain a replacement.  The supervisor must ensure that the defective respirator is replaced.

All workers wearing atmosphere-supplying respirator will work with a buddy.  If a worker experiences a malfunction of a respirator, he or she should signal to the buddy that he or she has had a respirator malfunction.  The buddy shall assist the worker in immediately exiting the work area.

Air Quality

For supplied air respirators, only Grade D breathing air shall be used in the cylinders.  The Program Administrator or his/her designee will coordinate refill of compressed air cylinders for SCBAs with an appropriate vendor.  {Business Name}  has no SCBA at this time.

Cleaning

Respirators are to be regularly cleaned and disinfected at a suitable location.  Respirators issued for the exclusive use of an employee shall be cleaned as often as necessary.  Atmosphere supplying and emergency use respirators are to be cleaned and disinfected after each use.

The following procedure is to be used when cleaning and disinfecting respirators:

· Disassemble respirator, removing any filters, canisters, or cartridges.

· Wash the facepiece and associated parts in a mild detergent with warm water.  Do not use organic solvents.

· Rinse completely in clean, warm water.

· Wipe the respirator with disinfectant wipes (70% Isopropyl Alcohol) to kill germs.

· Air-dry in a clean area.

· Reassemble the respirator and replace any defective part.

· Place in a clean, dry, plastic bag or other appropriate container.

Note:  The employee’s supervisor will ensure an adequate supply of appropriate cleaning and disinfectant at the cleaning station.  When supplies are low, employees should contact their supervisor who will order the needed supplies.

Inspection

All respirators used in routine situations will be inspected before each use and during cleaning.  All respirators maintained for use in emergency situations will be inspected at least monthly, and will be checked for proper function before and after each use.

Maintenance
Respirators are to be properly maintained at all times in order to ensure that they function properly and adequately protect the employee.  Maintenance involves a thorough visual inspecting for cleanliness and defects.  Worn or deteriorated parts will be replaced prior to use.  No components will be replaced or repairs made beyond those recommended by the manufacturer.  Repairs to regulators or alarms of atmosphere-supplying respirators will be conducted by a service agent licensed by the manufacture of the respirator.

The following checklist will be used when inspecting respirators:

Facepiece:

· Cracks, tears, or holes

· Facemask distortion

· Cracked or loose lenses/faceshield

Filters/Cartridges:
· Approval designation

· Gaskets

· Cracks or dents in housing

· Proper cartridge for hazard

Head straps:

· Breaks or tears

· Broken buckles

Valves:

· Residue or dirt

· Cracks or tears in valve material

Air Supply Systems:

· Breathing air quality/grade

· Condition of supply hoses

· Hose connections

· Settings on regulators and valves

Employees shall perform limited maintenance on their respirator in a designated area that is free of respiratory hazards.

Change schedules

Employees wearing air-purifying respirator (APR), N95, HEPA or N100 shall change the cartridges or mask when they first begin to experience difficulty breathing (i.e., resistance while wearing their masks.)

Chemical cartridges used for emergency response and chemical consolidation will be changed after each use.

Other chemical cartridges shall be changed-out in accordance with manufacturer’s recommendations and site-specific procedures.

Storage

Respirators must be stored in a clean, dry area, and in accordance with the manufacturer’s recommendations.  Each employee will clean and inspect their own APR in accordance with the provisions of this program and will store their respirator in a plastic bag in a suitable location.

Defective Respirator

Respirators that are defective or have defective parts shall be taken out of service and brought to {Individual responsible}.  If, during an inspection, an employee discovers a defect in a respirator, he/she is to bring the defect to the attention of his/her supervisor.  Supervisors will dispose of or give defective respirator to {Individual responsible}, who will decide whether to:

· Temporarily take the respirator out of service until it can be repaired.

· Perform a simple fix on the spot such as replacing a head strap.

· Dispose of the respirator due to an irreparable problem or defect.

When a respirator is taken out of service for an extended period of time, the respirator will be tagged “out of service”, and the employee will be given a replacement of similar make, model, and size.

Training

Training will be provided to users of respirators and their supervisors on the {Business Name} Respiratory Protection Program and their responsibilities under it, and on the OSHA Respiratory Protection Standard.  Workers will be trained prior to using a respirator in the workplace.  Supervisors will also be trained prior to using a respirator in the workplace or prior to supervising employees that must wear respirators.

The training course will cover the following topics:

· The {Business Name} Respiratory Protection Program and the OSHA Respiratory Protection standard.

· Respiratory hazards that may be encountered in their work areas and their health effects.

· Proper selection use and limitations of respirators.

· Respirator donning and user seal (fit)checks.

· Fit testing

· Emergency use procedures (if applicable).

· Maintenance and storage.

· Medical signs and symptoms limiting the effective use of respirators.

Employees will be retrained annually or as needed (e.g., if they change departments and need to use a different respirator).  Employees must demonstrate their understanding of the topics covered in the training through hands-on exercises.  Respirator training will be documented by {Individual responsible} and the documentation will include the type, model and size of respirator for which each employee has been trained and fit tested.

PROGRAM EVALUATION

{Individual responsible} will conduct periodic evaluations of the workplace to ensure that the provisions of this program are being implemented.  The evaluations will include periodic consultations with employees who use respirators and their supervisors, site inspections, air monitoring, and a review of records.

Problems identified will be noted.  The findings will be reported to the employee’s supervisor and will include specific corrective actions with target dates for the implementation of the corrections identified.

DOCUMENTATION AND RECORD KEEPING

A written copy of this program and the OSHA standard are kept in the {area kept} and is available to all employees who wish to review it.

{Individual responsible} maintains copies of training and fit test records.  These records will be updated as new employees are trained, as existing employees receive refresher training, and as new fit tests are conducted.

{Individual responsible} will also maintain copies of the medical records for all employees covered under the respirator program.  The completed medical questionnaire and documented findings are confidential and will remain with {where documents stored}.  Supervisors will only retain written recommendation regarding each employee’s ability to wear a respirator.

Appendix A

{Business Name}  VOLUNTARY RESPIRATOR USE PROGRAM

PURPOSE

The respiratory protection program at {Business Name} is intended to protect employees against recognized health hazards.  However, some employees may be irritated by the presence of non-hazardous air contaminants (such as pollen or animal dander).  When use of a respirator will help to alleviate irritation and when the respirator itself is judged to pose no additional risk to the wearer, {Business Name} employees will be allowed to voluntarily use respirators for comfort reasons.  This Program describes responsibilities and procedures for obtaining approval and medical clearance for voluntary respirator use.

RESPONSIBILITIES

Departments with employees who desire to use respiratory protection for comfort reasons shall:

· Notify {Individual responsible} with the names and social security numbers of affected employees as well as the manufacturer and style of the proposed respirators.

· Ensure that masks are not dirty or contaminated.

· Ensure that use of respirators does not interfere with employee’s ability to work safely.

· Provide information contained in Attachment D of this Program to all employees who voluntarily use respirators.

· Report any problems to {Individual responsible}.

· Ensure that respirators are not used without the approval of {Individual responsible}.

Employees who choose to use respiratory protection shall:

· Obtain permission from their department and {Individual responsible} to use a respirator.

· Read and understand the information contained in Attachment D of this Program.

· Inspect their respirators before each use, and clean them after each use.

· Report any problems to their supervisor.

{Individual responsible} shall:

· Approve all voluntary respirator use to ensure that selected respirators will not pose an additional risk to employees.  Only NIOSH approved respirators will be allowed.

· Determine if medical clearance is necessary for voluntary respirator users based on the type of respirator selected.

· Maintain records of all voluntary respirator use at {Business Name}
· Conduct periodic reviews of departmental policies on respirator users based on the type of respirator storage and use.

The {Individual responsible} shall conduct health screening of voluntary respirator wearers if it is determined that the selected respirator may cause physical stress for the wearer.

PROCEDURES

In their jobs at {Business Name}, some employee may perform tasks, which cause respiratory discomfort but which do not pose a health risk.  In these cases, {Individual responsible} will not require that respiratory protection be worn, but individuals may request that their departments provide respirators.  Individual employees who choose to use respiratory protection for comfort reasons (such as allergy prevention) must obtain permission for {Individual responsible}.  Departments which provide comfort respirators for their employees must furnish {Individual responsible} information about the types of respirators being used, the tasks being performed while respirators are worn, and the names and social security numbers of employee wearing the respirators.

HEALTH EVALUATION

The use of some respirators may impose an additional stress on the worker.  If an {individual responsible} representative determines that a selected respirator may cause physical stress, he or she may recommend another type of respirator or provide a medical clearance questionnaire to the person wanting to use the respirator.  Employees who receive medical clearance questionnaires must fill them out and send them to {Individual responsible}.  {Individual responsible} will determine if the respirator poses a risk to the employee, and whether or not the employee is physically able to perform assigned tasks while wearing a respirator.

RESPIRATOR SELECTION

Comfort respirators will be approved only when they are appropriate for protection against the irritating substance and when they pose no health risk to the wearer.  {Individual responsible} will make alternate recommendations if the selected respirator is inappropriate for the proposed use.

FIT TESTING

Fit testing is not required for voluntary respirator wearers.  However, individuals voluntarily wearing tight-fitting respirators may request that {Individual responsible} perform a fit test to ensure that the respirator achieves an effective mask-to-face seal.

MAINTENANCE

Each respirator user will have the responsibility for maintaining his/her own respirator.  Parts should be replaced when needed.  Respirators provided by departments for non-routine, general use will be maintained by the supervisor or a competent designated person.

INSPECTION & REPAIR

All respirators will be inspected before and after each day’s use by the respirator wearer.  Respirators needing repair will be repaired by the competent designated person using approved replacement parts only.  Repairs must be made in a manner that maintains the respirator’s NIOSH approval.

CLEANING, DISINFECTION AND STORAGE

Respirators will be cleaned and disinfected after each use according to procedures for specific respirators.  Respirators will be stored away from any contamination in a clean, sanitary place, on a flat surface in a sealed container.  Respirators must not be exposed to extreme temperatures.  It is not acceptable to hang a respirator by its straps.

TRAINING

Employees who choose to use respiratory protection must be familiar with the information contained in Attachment D.

APPENDIX B

Respiratory – Use Requirements Flowchart

29 CFR 1910.134c.



[image: image1]
Attachment C


 {Business Name}
Respirator Protocol


[image: image2]

[image: image3]
Attachment C


 
 Attachment D

MANDATORY INFORMATION FOR EMPLOYEES USING RESPIRATORS WHEN NOT REQUIRED UNDER STANDARD 1910.134

Respirators are an effective method of protection against designated hazards when properly selected and worn.  Respirator use is encouraged, even when exposures are below the exposure limit, to provide an additional level of comfort and protection for workers.  However, if a respirator is used improperly or not kept clean, the respirator itself can become a hazard to the worker.  Sometimes, workers may wear respirators to avoid exposures to hazards, even if the amount of hazardous substance does not exceed the limits set by OSHA standards.  If your employer provides respirators for your voluntary use, or if you provide your own respirator, you need to take certain precautions to be sure that the respirator itself does not present a hazard.

You should do the following:

1. Read and heed all instructions provided by the manufacturer on use, maintenance, cleaning and care, and warnings regarding the respirators limitations.

2. Choose respirators certified for use to protect against the contaminant of concern, NIOSH, the National Institute for Occupational Safety and Health of the U.S. Department of Health and Human Services, certifies respirators.  A label or statement of certification should appear on the respirator or respirator packaging.  It will tell you what the respirator is designed for and how much it will protect you.

3. Do not wear your respirator into atmospheres containing contaminants for which your respirator is not designed to protect against.  For example, a respirator designed to filter dust particles will not protect you against gases, vapors, or very small solid particles of fumes or smoke.

4. Keep track of your respirator so that you do not mistakenly use someone else’s respirator.
Each facility will need to decide who is going to be in charge of the areas that have red brackets.
For the medical evaluations, you can put in Licensed Health Care Provider (LHCP) if you are not doing them on site.  The medical evaluation may be stored with the LHCP off site, all you need to have is the determination if they can wear a respirator or not.  Remember, if you do them on site, they must be stored in a secure area with limited access and not with personnel files.

You will need to read this policy in full and make sure it works for you; every boiler plate policy has to be tailored to your specific site.
Are respirators:


 	Necessary to protect the health of the employee; OR


	Required by the employer





Must establish and implement a written respirator program with work-site specific procedures.





 No





  Yes





Does the employer permit voluntary use of respirators?





   No





  Stop	





  Yes





Does the only use of respirators involve the voluntary use of filtering face pieces (dust masks)?





  Yes





Employer determines that the respirator itself does not create a hazard





Must provide users with information contained in Attachment D.





No respirator program required.





.





  No





Employer determines that the respirator itself does not create a hazard.





Must provide users with information contained in Attachment D.





Must establish and implement those elements of a written respirator program necessary to  ensure that employee is medically able to use that respirator.











Current User on Program





New user to Program





Not Approved








Stop





Supervisors contact {Individual name}





Refit on different respirator





Fail





Stop





Fail on all respirators





Give storage and cleaning instructions





Pass





{Individual name} reviews worksite for respiratory hazard





Issue respirator





Pass





Stop





Fail on all respirators





Fit Test








Readjust or try different respirator





Fail





Begin fit demonstration and fit check





Fit test appointment made by employee, supervisor, or safety.





Conducted by {Individual name}.





Medical evaluation conducted by {Individual name} to determine respiratory fitness





Approved





Current User on Program





Not Approved








Stop








